Modified Top-down Clean Room Procedure: JHS-259

- Wash hands

- Dry hands with paper towel, discard paper towel.

- Put on gloves (note: it may be easier to put on a size larger than the size you normally use).

- Get two shoe cots (“booties”).

- Sit on bench with both shoes on “Dirty” side.

- Swing one leg up, put on shoe cot, place shoe on “Clean” side.

  - Note: if you touch the bottom of your shoe, change gloves

- Repeat with other shoe

- Remove gloves.  Put on new pair of gloves.

- Put on hairnet

- Put on safety glasses

- Put on lab coat

- Remove gloves

- Put on new pair of gloves